=======================================================================SIGMA1

SIGMA1

PROGRAM SIGMA1                                                    SIGMA1

==============                                                    SIGMA1

VERSION 73-1 (MARCH 1973)                                         SIGMA1

VERSION 76-1 (FEBRUARY 1976)                                      SIGMA1

VERSION 76-2 (OCTOBER 1976)                                       SIGMA1

VERSION 77-1 (JANUARY 1977)                                       SIGMA1

VERSION 78-1 (JULY 1978)                                          SIGMA1

VERSION 79-1 (JULY 1979)    CDC-7600 AND CRAY-1 VERSION.          SIGMA1

VERSION 80-1 (MAY 1980)     IBM, CDC AND CRAY VERSION             SIGMA1

VERSION 80-2 (DECEMBER 1980)IMPROVED BASED ON USER COMMENTS.      SIGMA1

VERSION 81-1 (MARCH 1981)   DOUBLE PRECISION IBM VERSION          SIGMA1

VERSION 81-2 (AUGUST 1981)  IMPROVED IBM SPEED AND STABILITY      SIGMA1

VERSION 82-1 (JANUARY 1982) IMPROVED COMPUTER COMPATIBILITY       SIGMA1

VERSION 83-1 (JANUARY 1983)*MAJOR RE-DESIGN.                      SIGMA1

*PAGE SIZE INCREASED - 1002 TO 2004.   SIGMA1

*ELIMINATED COMPUTER DEPENDENT CODING. SIGMA1

*NEW, MORE COMPATIBLE I/O UNIT NUMBER. SIGMA1

*ADDED STANDARD ALLOWABLE ERROR OPTION SIGMA1

(CURRENTLY 0.1 PER-CENT).             SIGMA1

*UNRESOLVED RESONANCE REGION COPIED.   SIGMA1

*1/V EXTENSION OF CROSS SECTIONS       SIGMA1

OUTSIDE OF TABULATED ENERGY RANGE AND SIGMA1

INTO UNRESOLVED ENERGY RANGE.         SIGMA1

VERSION 83-2 (OCTOBER 1983)*IMPROVED BASED ON USER COMMENTS.      SIGMA1

VERSION 84-1 (APRIL 1984)  *IMPROVED NUMERICAL STABILITY.         SIGMA1

*PARTIAL EVALUATION TREATMENT.         SIGMA1

VERSION 85-1 (APRIL 1985)  *ITERATE TO CONVERGENCE (USING THE SAMESIGMA1

ENERGY GRID FOR HOT CROSS SECTION AS  SIGMA1

COLD CROSS SECTIONS WAS FOUND TO BE   SIGMA1

INACCURATE).                          SIGMA1

*NEW FASTER HIGH ENERGY BROADENING.    SIGMA1

*UPDATED FOR ENDF/B-6  FORMATS.        SIGMA1

*SPECIAL I/O ROUTINES TO GUARANTEE     SIGMA1

ACCURACY OF ENERGY.                   SIGMA1

*DOUBLE PRECISION TREATMENT OF ENERGY  SIGMA1

(REQUIRED FOR NARROW RESONANCES).     SIGMA1

VERSION 85-2 (AUGUST 1985) *FORTRAN-77/H VERSION                  SIGMA1

VERSION 86-1 (JANUARY 1986)*ENERGY DEPENDENT SCATTERING RADIUS    SIGMA1

VERSION 88-1 (JULY 1988)   *OPTION...INTERNALLY DEFINE ALL I/O    SIGMA1

FILE NAMES (SEE, SUBROUTINE FILEIO    SIGMA1

FOR DETAILS).                         SIGMA1

*IMPROVED BASED ON USER COMMENTS.      SIGMA1

VERSION 89-1 (JANUARY 1989)*PSYCHOANALYZED BY PROGRAM FREUD TO    SIGMA1

INSURE PROGRAM WILL NOT DO ANYTHING   SIGMA1

CRAZY.                                SIGMA1

*UPDATED TO USE NEW PROGRAM CONVERT    SIGMA1

KEYWORDS.                             SIGMA1

*ADDED LIVERMORE CIVIC COMPILER        SIGMA1

CONVENTIONS.                          SIGMA1

VERSION 90-1 (JUNE 1990)   *UPDATED BASED ON USER COMMENTS        SIGMA1

*ADDED FORTRAN SAVE OPTION             SIGMA1

*NEW MORE CONSISTENT ENERGY OUTPUT     SIGMA1

ROUTINES                              SIGMA1

VERSION 91-1 (JULY 1991)   *WARNING...INPUT PARAMETER FORMAT      SIGMA1

HAS BEEN CHANGED - SEE BELOW FOR      SIGMA1

DETAILS.                              SIGMA1

*ADDED CHARGED PARTICLE PROJECTILES    SIGMA1

*OUTPUT ENERGY RANGE IS ALWAYS AT      SIGMA1

LEAST AS LARGE AS INPUT ENERGY RANGE. SIGMA1

*NO 1/V EXTENSION OF CROSS SECTIONS    SIGMA1

FROM UNRESOLVED ENERGY RANGE.         SIGMA1

VERSION 92-1 (JANUARY 1992)*INSURE MINIMUM AND MAXIMUM CROSS      SIGMA1

SECTIONS ARE ALWAYS KEPT (NOT THINNED)SIGMA1

*MT=19 (FIRST CHANCE FISSION) TREATED  SIGMA1

THE SAME AS FISSION.                  SIGMA1

*VARIABLE MINIMUM CROSS SECTION OF     SIGMA1

INTEREST - TO ALLOW SMALL CROSS       SIGMA1

SECTIONS NEAR THRESHOLDS TO BE        SIGMA1

TREATED PROPERLY.                     SIGMA1

*ALL ENERGIES INTERNALLY ROUNDED PRIOR SIGMA1

TO CALCULATIONS.                      SIGMA1

*COMPLETELY CONSISTENT I/O AND ROUNDINGSIGMA1

ROUTINES - TO MINIMIZE COMPUTER       SIGMA1

DEPENDENCE.                           SIGMA1

VERSION 92-2 (JULY 1992)   *CORRECTED BUG ASSOCIATED WITH         SIGMA1

THRESHOLD REACTIONS.                  SIGMA1

*UNRESOLVED REGION COPIED WITHOUT      SIGMA1

THINNING (IT SHOULD BE EXACTLY THE    SIGMA1

SAME AT ALL TEMPERATURES).            SIGMA1

*NO THINNING OF REACTIONS (MT) THAT    SIGMA1

WERE NOT BROADENED.                   SIGMA1

VERSION 93-1 (APRIL 1993)  *INCREASED PAGE SIZE FROM 2004         SIGMA1

TO 24000 ENERGY PONTS.                SIGMA1

VERSION 94-1 (JANUARY 1994)*VARIABLE ENDF/B DATA FILENAMES        SIGMA1

TO ALLOW ACCESS TO FILE STRUCTURES    SIGMA1

(WARNING - INPUT PARAMETER FORMAT     SIGMA1

HAS BEEN CHANGED)                     SIGMA1

*CLOSE ALL FILES BEFORE TERMINATING    SIGMA1

(SEE, SUBROUTINE ENDIT)               SIGMA1

VERSION 96-1 (JANUARY 1996) *COMPLETE RE-WRITE                    SIGMA1

*IMPROVED COMPUTER INDEPENDENCE       SIGMA1

*ALL DOUBLE PRECISION                 SIGMA1

*ON SCREEN OUTPUT                     SIGMA1

*UNIFORM TREATMENT OF ENDF/B I/O      SIGMA1

*IMPROVED OUTPUT PRECISION            SIGMA1

*DEFINED SCRATCH FILE NAMES           SIGMA1

*ALWAYS INCLUDE THERMAL VALUE         SIGMA1

VERSION 97-1 (APRIL 1997)   *OPTIONALLY SET NEGATIVE CROSS        SIGMA1

SECTIONS = 0 ON INPUT AND            SIGMA1

OUTPUT.                              SIGMA1

*INCREASED PAGE SIZE FROM 24000       SIGMA1

TO 60000 ENERGY POINTS.              SIGMA1

VERSION 99-1 (MARCH 1999)   *CORRECTED CHARACTER TO FLOATING      SIGMA1

POINT READ FOR MORE DIGITS           SIGMA1

*UPDATED TEST FOR ENDF/B FORMAT       SIGMA1

VERSION BASED ON RECENT FORMAT CHANGESIGMA1

*TREAT LOW ENERGY INITIAL CROSS       SIGMA1

SECTIONS AS LOG-LOG INTERPOLABLE     SIGMA1

*CONSTANT (RATHER THAN 1/V) EXTENSION SIGMA1

TO HIGHER ENERGY.                    SIGMA1

*UPDATED CONSTANTS BASED ON CSEWG     SIGMA1

SUBCOMMITTEE RECOMMENDATIONS         SIGMA1

*GENERAL IMPROVEMENTS BASED ON        SIGMA1

USER FEEDBACK                        SIGMA1

VERSION 99-2 (JUNE 1999)    *EXTENDED RANGE OF INTEGRALS FROM 4   SIGMA1

TO 5 UNITS ON EACH SIDE OF ENERGY    SIGMA1

POINT TO ALLOW FOR LARGER VARIATION  SIGMA1

IN THE LOCAL CROSS SECTION           SIGMA1

*ASSUME ENDF/B-6, NOT 5, IF MISSING   SIGMA1

MF=1, MT-451.                        SIGMA1

VERSION 99-3 (OCTOBER 1999))*IMPROVED ERFC FUNCTION DEFINITION.   SIGMA1

I THANK BOB MACFARLANE (LANL) FOR    SIGMA1

SUPPLYING A MORE ACCURATE ERFC       SIGMA1

FUNCTION.                            SIGMA1

VERS. 2000-1 (FEBRUARY 2000)*CORRECTED LOW ENERGY INTERPOLATION   SIGMA1

FOR NON-POSITIVE CROSS SECTIONS      SIGMA1

*GENERAL IMPROVEMENTS BASED ON        SIGMA1

USER FEEDBACK                        SIGMA1

VERS. 2002-1 (MAY 2002)     *OPTIONAL INPUT PARAMETERS            SIGMA1

VERS. 2004-1 (JAN. 2004)    *OPTIONALLY IGNORE UNRESOLVED REGION  SIGMA1

*CORRECTED PROBLEM AT THE RESOLVED/   SIGMA1

UNRESOLVED ENERGY BOUNDARY.          SIGMA1

*CORRECTED HIGH ENERGY CONSTANT CROSS SIGMA1

SECTION EXTENSION.                   SIGMA1

*TIGHTER CRITERIA FOR INITIAL ENERGY  SIGMA1

POINT SPACING                        SIGMA1

*TEMPERATURE DEPENDENT ENERGY POINT   SIGMA1

SPACING.                             SIGMA1

*ADDED NEW REICH-MOORE (LRF=7) TO     SIGMA1

FILE2 TO ALLOW COPY TO FIND ANY      SIGMA1

FOLLOWING UNRESOLVED PARAMETERS      SIGMA1

VERS. 2005-1 (JUNE 2005)    *CORRECTED ERROR IN EHOT3 EQUIVALENCE SIGMA1

TO EHOT - THIS ONLY EFFECTS VERY BIG SIGMA1

OUTPUT FILES.                        SIGMA1

VERS. 2007-1 (JAN. 2007)    *CHECKED AGAINST ALL ENDF/B-6.        SIGMA1

*INCREASED PAGE SIZE FROM 60,000      SIGMA1

TO 360,000 ENERGY POINTS.            SIGMA1

VERS. 2008-1 (APRIL 2008)   *1/2 INITIAL ENERGY POINT SPACING     SIGMA1

*72 CHARACTER FILE NAMES.             SIGMA1

VERS. 2010-1 (Apr. 2010)    *ASSUME LOW ENERGY LOG-LOG VARIATION  SIGMA1

UP TO 1/A (eV) FOR ALL BUT TOTAL AND SIGMA1

ELASTIC.                             SIGMA1

*CHANGED DEFAULT UNCERTAINTY TO 0.01% SIGMA1

FROM 0.1%                            SIGMA1

*ALLOW MULTIPLE, ADJACENT UNRESOLVED  SIGMA1

RESONANCE REGIONS = COMBINE INTO ONE SIGMA1

LARGER ENERGY RANGE TO COPY.         SIGMA1

*DO NOT BROADEN SECTIONS THAT START   SIGMA1

ABOVE 1 MILLION KT - PREVIOUSLY IT   SIGMA1

WAS ASSUMED TOTAL, ELASTIC, CAPTURE  SIGMA1

AND FISSION, AND LARGE SECTIONS (OVERSIGMA1

10,000 ENERGY POINTS) WOULD BROADEN. SIGMA1

VERS. 2012-1 (Aug. 2012)    *CHANGE COPY CRITERIA TO HANDLE NEW   SIGMA1

(N,N') DATA = THRESHOLD MAY BE VERY  SIGMA1

HIGH (OLD CRITERIA) BUT INCLUDES MANYSIGMA1

TABULATED ENERGY POINTS (NEW ADDED   SIGMA1

CRITERIA).                           SIGMA1

*ADDED STOP IF INCIDENT PARTICLE DATA SIGMA1

CANNOT BE DOPPLER BROADENED, E.G.,   SIGMA1

PHOTON INCIDENT.                     SIGMA1

*Added CODENAME                       SIGMA1

*32 and 64 bit Compatible             SIGMA1

*Added ERROR stop                     SIGMA1

VERS. 2013-1 (Nov. 2013)    *Added NO broadening above 10 MeV -   SIGMA1

this is to handle newer evaluations  SIGMA1

that extend to higher energies and   SIGMA1

may do "strange" things to stop one  SIGMA1

MT and then include it as part of    SIGMA1

a sum at higher energies, e.g. this  SIGMA1

change will copy ALL points above    SIGMA1

10 MeV, thus avoiding problems near  SIGMA1

transistion energies at 20. 30, etc. SIGMA1

MeV or higher energies.              SIGMA1

VERS. 2015-1 (Jan. 2015)    *Replaced ALL 3 way IF Statements.    SIGMA1

*Replaced ALL LOGICAL by INTEGER.     SIGMA1

*Extended OUT9.                       SIGMA1

VERS. 2017-1 (May  2017)    *For MF=2 only use MT=151 = Defines   SIGMA1

Unresolved Resonance Region (URR).   SIGMA1

Ignore - NJOY created MT=152 and 153.SIGMA1

*Increased page size to 1,2000,000.   SIGMA1

*All floating input parameters changedSIGMA1

to character input + IN9 conversion. SIGMA1

*Added NRO = energy dependent scatter SIGMA1

radius to copying FILE2 parameters   SIGMA1

to define unresolved energy range.   SIGMA1

*Corrected energy dependent scatteringSIGMA1

radius for all resonance types (see, SIGMA1

the above comments).                 SIGMA1

VERS. 2018-1 (Nov. 2018)    *Added on-line report for ALL ENDERRORSIGMA1

VERS. 2019-1 (June 2019)    *Terminate if MF=3 Point Count and    SIGMA1

Interpolation Law do not agree.      SIGMA1

*Terminate if MF=3 Background         SIGMA1

Interpolation is NOT Linear.         SIGMA1

*Terminate if MF/MT=1/451 Input       SIGMA1

temperature exceeds requested        SIGMA1

Temperature - otherwise the output   SIGMA1

by this code to MF=3 would appear    SIGMA1

to be at the WRONG temperature.      SIGMA1

*Additional Interpolation Law Tests   SIGMA1

*Check consistency of Maximum         SIGMA1

Tabulated cross sections for ALL MT  SIGMA1

processed - print WQARNING if NOT    SIGMA1

the same for ALL MTs.                SIGMA1

VERS. 2020-1 (Dec. 2020)    *Complete Re-write of convergence     SIGMA1

*Replaced INCORE9 by INCORE10.        SIGMA1

*Updated minimum/maximum convergence  SIGMA1

procedure.                           SIGMA1

*Added Target Isomer State            SIGMA1

*Check Atomic Weight > 0              SIGMA1

VERS. 2021-1 (Mar. 2021)    *Updated for FORTRAN 2018             SIGMA1

*Minimum Cross Section is no longer   SIGMA1

an input option - set to 1.0d-30.    SIGMA1

VERS. 2023-1 (Feb. 2023)    *Decreased page size from 1,200,000   SIGMA1

to 120,000.                          SIGMA1

SIGMA1

OWNED, MAINTAINED AND DISTRIBUTED BY                              SIGMA1

------------------------------------                              SIGMA1

THE NUCLEAR DATA SECTION                                          SIGMA1

INTERNATIONAL ATOMIC ENERGY AGENCY                                SIGMA1

P.O. BOX 100                                                      SIGMA1

A-1400, VIENNA, AUSTRIA                                           SIGMA1

EUROPE                                                            SIGMA1

SIGMA1

ORIGINALLY WRITTEN BY                                             SIGMA1

------------------------------------                              SIGMA1

Dermott E. Cullen                                                 SIGMA1

SIGMA1

PRESENT CONTACT INFORMATION                                       SIGMA1

---------------------------                                       SIGMA1

Dermott E. Cullen                                                 SIGMA1

1466 Hudson Way                                                   SIGMA1

Livermore, CA 94550                                               SIGMA1

U.S.A.                                                            SIGMA1

Telephone  925-443-1911                                           SIGMA1

E. Mail    RedCullen1@Comcast.net                                 SIGMA1

Website    RedCullen1.nedt/HOMEPAGE.NEW                           SIGMA1

SIGMA1

Acknowledgement 2004                                              SIGMA1

--------------------                                              SIGMA1

Currently almost all improvements to this code are based upon     SIGMA1

feedback from code users who report problems. This feedback       SIGMA1

benefits ALL users of this code, and ALL users are encouraged     SIGMA1

to report problems.                                               SIGMA1

SIGMA1

Improvements on the 2004 version of this code based on user       SIGMA1

feedback including,                                               SIGMA1

1) Bret Beck  - reported a problem at the resolved/unresolved     SIGMA1

energy boundary.                                  SIGMA1

2) S. Ganesan - reported a problem for small temperature changes. SIGMA1

SIGMA1

AUTHORS MESSAGE                                                   SIGMA1

---------------                                                   SIGMA1

THE REPORT DESCRIBED ABOVE IS THE LATEST PUBLISHED DOCUMENTATION  SIGMA1

FOR THIS PROGRAM. HOWEVER, THE COMMENTS BELOW SHOULD BE CONSIDEREDSIGMA1

THE LATEST DOCUMENTATION INCLUDING ALL RECENT IMPROVEMENTS. PLEASESIGMA1

READ ALL OF THESE COMMENTS BEFORE IMPLEMENTATION, PARTICULARLY    SIGMA1

THE COMMENTS CONCERNING MACHINE DEPENDENT CODING.                 SIGMA1

SIGMA1

AT THE PRESENT TIME WE ARE ATTEMPTING TO DEVELOP A SET OF COMPUTERSIGMA1

INDEPENDENT PROGRAMS THAT CAN EASILY BE IMPLEMENTED ON ANY ONE    SIGMA1

OF A WIDE VARIETY OF COMPUTERS. IN ORDER TO ASSIST IN THIS PROJECTSIGMA1

IT WOULD BE APPECIATED IF YOU WOULD NOTIFY THE AUTHOR OF ANY      SIGMA1

COMPILER DIAGNOSTICS, OPERATING PROBLEMS OR SUGGESTIONS ON HOW TO SIGMA1

IMPROVE THIS PROGRAM. HOPEFULLY, IN THIS WAY FUTURE VERSIONS OF   SIGMA1

THIS PROGRAM WILL BE COMPLETELY COMPATIBLE FOR USE ON YOUR        SIGMA1

COMPUTER.                                                         SIGMA1

SIGMA1

PURPOSE                                                           SIGMA1

-------                                                           SIGMA1

THIS PROGRAM IS DESIGNED TO DOPPLER BROADEN NEUTRON INDUCED       SIGMA1

CROSS SECTIONS. EACH SECTION OF CROSS SECTIONS (FILE 3) IS READ   SIGMA1

FROM THE ENDF/B FORMAT. THE DATA IS DOPPLER BROADENED, THINNED    SIGMA1

AND OUTPUT IN THE ENDF/B FORMAT.                                  SIGMA1

SIGMA1

IN THE FOLLOWING DISCUSSION FOR SIMPLICITY THE ENDF/B TERMINOLOGY SIGMA1

---ENDF/B TAPE---WILL BE USED. IN FACT THE ACTUAL MEDIUM MAY BE   SIGMA1

TAPE, CARDS, DISK OR ANY OTHER MEDIUM.                            SIGMA1

SIGMA1

ENDF/B FORMAT                                                     SIGMA1

-------------                                                     SIGMA1

THIS PROGRAM ONLY USES THE ENDF/B BCD OR CARD IMAGE FORMAT (AS    SIGMA1

OPPOSED TO THE BINARY FORMAT) AND CAN HANDLE DATA IN ANY VERSION  SIGMA1

OF THE ENDF/B FORMAT (I.E., ENDF/B-1, 2, 3, 4, 5, 6 FORMAT).      SIGMA1

SIGMA1

IT IS ASSUMED THAT THE DATA IS CORRECTLY CODED IN THE ENDF/B      SIGMA1

FORMAT AND NO ERROR CHECKING IS PERFORMED. IN PARTICULAR IT IS    SIGMA1

ASSUMED THAT THE MAT, MF AND MT ON EACH CARD IS CORRECT. SEQUENCE SIGMA1

NUMBERS (COLUMNS 76-80) ARE IGNORED ON INPUT, BUT WILL BE         SIGMA1

CORRECTLY OUTPUT ON ALL CARDS. THE FORMAT OF SECTION MF=1, MT=451 SIGMA1

AND ALL SECTIONS OF MF=3 MUST BE CORRECT. THE PROGRAM COPIES ALL  SIGMA1

OTHER SECTION OF DATA AS HOLLERITH AND AS SUCH IS INSENSITIVE TO  SIGMA1

THE CORRECTNESS OR INCORRECTNESS OF ALL OTHER SECTIONS.           SIGMA1

SIGMA1

ALL CROSS SECTIONS THAT ARE USED BY THIS PROGRAM MUST BE TABULATEDSIGMA1

AND LINEARLY INTERPOLABLE IN ENERGY AND CROSS SECTION (ENDF/B     SIGMA1

INTERPOLATION LAW 2). FILE 3 CROSS SECTIONS MAY BE MADE LINEARLY  SIGMA1

INTERPOLABLE BY USING PROGRAM LINEAR (UCRL-50400, VOL.17, PART A).SIGMA1

FILE 2 RESONANCE PARAMETERS MAY BE USED TO RECONSTRUCT ENERGY     SIGMA1

DEPENDENT CROSS SECTIONS AND ADD IN FILE 3 BACKGROUND CROSS       SIGMA1

SECTIONS TO DEFINE LINEARLY INTERPOLABLE CROSS SECTIONS BY USING  SIGMA1

PROGRAM RECENT (UCRL-50400, VOL. 17, PART C). IF THIS PROGRAM     SIGMA1

FINDS THAT THE FILE 3 CROSS SECTIONS ARE NOT LINEARLY INTERPOLABLESIGMA1

THIS PROGRAM WILL TERMINATE EXECUTION.                            SIGMA1

SIGMA1

UNRESOLVED RESONANCE REGION                                       SIGMA1

---------------------------                                       SIGMA1

IN THE UNRESOLVED RESONANCE REGION IT IS NOT POSSIBLE TO EXACTLY  SIGMA1

DEFINE THE ENERGY DEPENDENCE OF THE CROSS SECTIONS. THE AVERAGE   SIGMA1

WIDTHS AND SPACINGS GIVEN IN ENDF/B ARE ONLY ADEQUATE TO DEFINE   SIGMA1

AVERAGE VALUES OF THE CROSS SECTIONS. THEREFORE ALL CROSS SECTIONSSIGMA1

IN THE ENDF/B FORMAT FOR THE UNRESOLVED REGION ARE REALLY AVERAGE SIGMA1

VALUES WHICH CANNOT BE DOPPLER BROADENED USING THE SIGMA1 METHOD  SIGMA1

(WHICH REQUIRES TABULATED, LINEARLY INTERPOLABLE, ENERGY DEPENDENTSIGMA1

CROSS SECTIONS.                                                   SIGMA1

SIGMA1

THEREFORE,                                                        SIGMA1

(1) ALL TABULATED POINTS WITHIN THE UNRESOLVED RESONANCE REGION   SIGMA1

WILL BE COPIED, WITHOUT MODIFICATION OR BROADENING. ADOPTION OF   SIGMA1

THIS CONVENTION WILL ALLOW SUBSEQUENT PROGRAMS TO PROPERLY DEFINE SIGMA1

SELF-SHIELDED, DOPPLER BROADENED CROSS SECTIONS IN THE UNRESOLVED SIGMA1

RESONANCE REGION.                                                 SIGMA1

(2) CROSS SECTIONS WILL BE EXTENDED AS 1/V ABOVE THE UPPER ENERGY SIGMA1

LIMIT OF THE RESOLVED RESONANCE REGION AND BELOW THE LOWER ENERGY SIGMA1

LIMIT OF THE CONTINUUUM REGION (I.E. INTO THE UNRESOLVED          SIGMA1

RESONANCE REGION). THIS CONVENTION WILL GUARANTEE A SMOOTH        SIGMA1

BEHAVIOR CLOSE TO THE UNRESOLVED RESONANCE REGION BOUNDARIES.     SIGMA1

SIGMA1

OUTPUT FORMAT                                                     SIGMA1

-------------                                                     SIGMA1

IN THIS VERSION OF SIGMA1 ALL FILE 3 ENERGIES WILL BE OUTPUT IN   SIGMA1

F (INSTEAD OF E) FORMAT IN ORDER TO ALLOW ENERGIES TO BE WRITTEN  SIGMA1

WITH UP TO 9 DIGITS OF ACCURACY. IN PREVIOUS VERSIONS THIS WAS AN SIGMA1

OUTPUT OPTION. HOWEVER USE OF THIS OPTION TO COMPARE THE RESULTS  SIGMA1

OF ENERGIES WRITTEN IN THE NORMAL ENDF/B CONVENTION OF 6 DIGITS   SIGMA1

TO THE 9 DIGIT OUTPUT FROM THIS PROGRAM DEMONSTRATED THAT FAILURE SIGMA1

TO USE THE 9 DIGIT OUTPUT CAN LEAD TO LARGE ERRORS IN THE DATA    SIGMA1

JUST DUE TO TRANSLATION OF THE ENERGIES TO THE ENDF/B FORMAT.     SIGMA1

SIGMA1

CONTENTS OF OUTPUT                                                SIGMA1

------------------                                                SIGMA1

ENTIRE EVALUATIONS ARE OUTPUT, NOT JUST THE BROADENED FILE 3      SIGMA1

CROSS SECTIONS, E.G. ANGULAR AND ENERGY DISTRIBUTIONS ARE ALSO    SIGMA1

INCLUDED.                                                         SIGMA1

SIGMA1

DOCUMENTATION                                                     SIGMA1

-------------                                                     SIGMA1

THE FACT THAT THIS PROGRAM HAS OPERATED ON THE DATA IS DOCUMENTED SIGMA1

BY THE ADDITION OF THREE COMMENTS CARDS AT THE END OF EACH        SIGMA1

HOLLERITH SECTION IN THE FORM                                     SIGMA1

SIGMA1

***************** PROGRAM SIGMA1 (2023-1) ***************         SIGMA1

DATA DOPPLER BROADENED TO 300.0   KELVIN AND                      SIGMA1

DATA THINNED TO WITHIN AN ACCURACY OF  0.1 PER-CENT               SIGMA1

SIGMA1

THE ORDER OF ALL SIMILAR COMMENTS (FROM LINEAR,RECENT AND GROUPY) SIGMA1

REPRESENTS A COMPLETE HISTORY OF ALL OPERATIONS PERFORMED ON      SIGMA1

THE DATA.                                                         SIGMA1

SIGMA1

THESE COMMENT CARDS ARE ONLY ADDED TO EXISTING HOLLERITH SECTIONS,SIGMA1

I.E., THIS PROGRAM WILL NOT CREATE A HOLLERITH SECTION. THE FORMATSIGMA1

OF THE HOLLERITH SECTION IN ENDF/B-5 DIFFERS FROM THE THAT OF     SIGMA1

EARLIER VERSIONS OF ENDF/B. BY READING AN EXISTING MF=1, MT=451   SIGMA1

IT IS POSSIBLE FOR THIS PROGRAM TO DETERMINE WHICH VERSION OF     SIGMA1

THE ENDF/B FORMAT THE DATA IS IN. WITHOUT HAVING A SECTION OF     SIGMA1

MF=1, MT=451 PRESENT IT IS IMPOSSIBLE FOR THIS PROGRAM TO         SIGMA1

DETERMINE WHICH VERSION OF THE ENDF/B FORMAT THE DATA IS IN, AND  SIGMA1

AS SUCH IT IS IMPOSSIBLE FOR THE PROGRAM TO DETERMINE WHAT FORMAT SIGMA1

SHOULD BE USED TO CREATE A HOLLERITH SECTION.                     SIGMA1

SIGMA1

REACTION INDEX                                                    SIGMA1

--------------                                                    SIGMA1

THIS PROGRAM DOES NOT USE THE REACTION INDEX WHICH IS GIVEN IN    SIGMA1

SECTION MF=1, MT=451 OF EACH EVALUATION.                          SIGMA1

SIGMA1

THIS PROGRAM DOES NOT UPDATE THE REACTION INDEX IN MF=1, MT=451.  SIGMA1

THIS CONVENTION HAS BEEN ADOPTED BECAUSE MOST USERS DO NOT        SIGMA1

REQUIRE A CORRECT REACTION INDEX FOR THEIR APPLICATIONS AND IT WASSIGMA1

NOT CONSIDERED WORTHWHILE TO INCLUDE THE OVERHEAD OF CONSTRUCTING SIGMA1

A CORRECT REACTION INDEX IN THIS PROGRAM. HOWEVER, IF YOU REQUIRE SIGMA1

A REACTION INDEX FOR YOUR APPLICATIONS, AFTER RUNNING THIS PROGRAMSIGMA1

YOU MAY USE PROGRAM DICTIN TO CREATE A CORRECT REACTION INDEX.    SIGMA1

SIGMA1

SECTION SIZE                                                      SIGMA1

------------                                                      SIGMA1

SINCE THIS PROGRAM USES A LOGICAL PAGING SYSTEM THERE IS NO LIMIT SIGMA1

TO THE NUMBER OF POINTS IN ANY SECTION, E.G., THE TOTAL CROSS     SIGMA1

SECTION MAY BE REPRESENTED BY 200,000 DATA POINTS.                SIGMA1

SIGMA1

SELECTION OF DATA                                                 SIGMA1

-----------------                                                 SIGMA1

THE PROGRAM SELECTS MATERIALS TO BE BROADENED BASED EITHER ON     SIGMA1

MAT (ENDF/B MAT NO.) OR ZA. THE PROGRAM ALLOWS UP TO 100 MAT OR   SIGMA1

ZA RANGES TO BE SPECIFIED. THE PROGRAM WILL ASSUME THAT THE       SIGMA1

ENDF/B TAPE IS IN EITHER MAT OR ZA ORDER, WHICHEVER CRITERIA IS   SIGMA1

USED TO SELECT MATERIALS, AND WILL TERMINATE WHEN A MAT OR ZA     SIGMA1

IS FOUND THAT IS ABOVE THE RANGE OF ALL REQUESTS.                 SIGMA1

SIGMA1

ENERGY GRID OF BROADENED DATA                                     SIGMA1

-----------------------------                                     SIGMA1

THE ENERGY GRID FOR THE DOPPLER BROADENED CROSS SECTIONS IS       SIGMA1

SELECTED TO INSURE THAT THE BROADENED DATA IS LINEAR-LINEAR       SIGMA1

INTERPOLABLE. AS SUCH THE ENERGY GRID FOR THE BROADENED DATA      SIGMA1

MAY NOT BE THE SAME AS THE ENERGY GRID FOR THE ORIGINAL           SIGMA1

UNBROADENED DATA. GENERALLY AFTER BROADENING THERE WILL BE        SIGMA1

FEWER DATA POINTS IN THE RESONANCE REGION, BUT AT LOW ENERGY      SIGMA1

THERE MAY BE MORE POINTS, DUE TO THE 1/V LOW ENERGY EFFECT        SIGMA1

CREATED BY DOPPLER BROADENING.                                    SIGMA1

SIGMA1

EFFECTIVE TEMERATURE INCREASE                                     SIGMA1

-----------------------------                                     SIGMA1

IF THE ORIGINAL DATA IS NOT AT ZERO KELVIN THE PROGRAM WILL       SIGMA1

BROADEN THE DATA BY THE EFFECTIVE TEMPERATURE DIFFENCE TO THE     SIGMA1

FINAL TEMPERATURE. IF THE DATA IS ALREADY AT A TEMPERATURE THAT   SIGMA1

IS HIGHER THAN THE FINAL TEMPERATURE DOPPLER BROADENING IS        SIGMA1

NATURALLY NOT PERFORMED AND THE TEMPERATURE IN THE SECTION IS LEFTSIGMA1

AT ITS ORIGINAL VALUE.                                            SIGMA1

SIGMA1

MULTIPLE FINAL TEMPERATURES                                       SIGMA1

---------------------------                                       SIGMA1

THE PRESENT VERSION ONLY DOPPLER BROADENS TO ONE FINAL TEMPERATURESIGMA1

(IF THERE IS SUFFICIENT INTEREST EXPRESSED BY USERS FUTURE        SIGMA1

VERSION MAY BROADEN TO MULTIPLE TEMPERATURES. PLEASE              SIGMA1

CONTACT THE AUTHOR IF YOU ARE INTERESTED IN A MULTIPLE            SIGMA1

TEMPERATURE OPTION).                                              SIGMA1

SIGMA1

PROGRAM OPERATION                                                 SIGMA1

-----------------                                                 SIGMA1

EACH SECTION OF FILE 3 DATA IS CONSIDERED SEPERATELY. THE DATA    SIGMA1

IS READ AND DOPPLER BROADENED A PAGE AT A TIME (ONE PAGE IS       SIGMA1

60000 DATA POINTS). UP TO THREE PAGES OF DATA MAY BE IN THE CORE  SIGMA1

AT ANY GIVEN TIME, THE PAGE BEING BROADENED, THE PAGE BELOW IT    SIGMA1

IN ENERGY AND THE PAGE ABOVE IT IN ENERGY. AFTER A PAGE HAS BEEN  SIGMA1

BROADENED IT IS THINNED, IF THE ENTIRE SECTION CONTAINS ONLY      SIGMA1

ONE PAGE OR LESS, IT WILL STILL BE CORE RESIDENT AND WILL BE      SIGMA1

WRITTEN DIRECTLY FROM CORE TO THE OUTPUT TAPE. IF THE BROADENED,  SIGMA1

THINNED SECTION IS LARGER THAN A PAGE, AFTER A PAGE HAS BEEN      SIGMA1

BROADENED AND THINNED IT IS WRITTEN TO A SCRATCH FILE. AFTER THE  SIGMA1

ENTIRE SECTION HAS BEEN BROADENED AND THINNED THE DATA IS READ    SIGMA1

FROM SCRATCH TO CORE, ONE PAGE AT A TIME, THE OUTPUT TO THE OUTPUTSIGMA1

TAPE.                                                             SIGMA1

SIGMA1

ALLOWABLE ERROR                                                   SIGMA1

---------------                                                   SIGMA1

AFTER DOPPLER BROADENING THE CROSS SECTION IN THE RESONANCE REGIONSIGMA1

WILL GENERALLY BE MUCH SMOOTHER THAN THE UNBROADENED DATA AND CAN SIGMA1

BE REPRESENTED TO THE SAME ACCURACY BY A SMALLER NUMBER OF ENERGY SIGMA1

POINTS. THEREFORE AFTER DOPPLER BROADENING THE DATA CAN BE THINNEDSIGMA1

WITH ESSENTIALLY NO LOSE OF INFORMATION.                          SIGMA1

SIGMA1

THE ALLOWABLE ERROR MAY BE ENERGY INDEPENDENT (CONSTANT) OR ENERGYSIGMA1

DEPENDENT. THE ALLOWABLE ERROR IS DESCRIBED BY A TABULATED        SIGMA1

FUNCTION OF UP TO 20 (ENERGY,ERROR) PAIRS AND LINEAR INTERPOLATIONSIGMA1

BETWEEN TABULATED POINTS. IF ONLY ONE TABULATED POINT IS GIVEN THESIGMA1

ERROR WILL BE CONSIDERED CONSTANT OVER THE ENTIRE ENERGY RANGE.   SIGMA1

WITH THIS ENERGY DEPENDENT ERROR ONE MAY OPTIMIZE THE OUTPUT FOR  SIGMA1

ANY GIVEN APPLICATION BY USING A SMALL ERROR IN THE ENERGY RANGE  SIGMA1

OF INTEREST AND A LESS STRINGENT ERROR IN OTHER ENERGY RANGES.    SIGMA1

SIGMA1

INPUT FILES                                                       SIGMA1

-----------                                                       SIGMA1

UNIT  DESCRIPTION                                                 SIGMA1

----  -----------                                                 SIGMA1

2  INPUT CARDS (BCD - 80 CHARACTERS/RECORD)                    SIGMA1

10  ORIGINAL ENDF/B DATA (BCD - 80 CHARACTERS/RECORD)           SIGMA1

SIGMA1

OUTPUT FILES                                                      SIGMA1

------------                                                      SIGMA1

UNIT  DESCRIPTION                                                 SIGMA1

----  -----------                                                 SIGMA1

3  OUTPUT REPORT (BCD - 120 CHARACTERS/RECORD)                 SIGMA1

11  FINAL ENDF/B DATA (BCD - 80 CHARACTERS/RECORD)              SIGMA1

SIGMA1

SCRATCH FILES                                                     SIGMA1

-------------                                                     SIGMA1

UNIT  DESCRIPTION                                                 SIGMA1

----  -----------                                                 SIGMA1

12  SCRATCH FILE FOR BROADENED DATA                             SIGMA1

(BINARY - 180000 WORDS/RECORD - DOUBLE PRECISION/           SIGMA1

42000 WORDS/RECORD - SINLGE PRECISION)           SIGMA1

SIGMA1

OPTIONAL STANDARD FILE NAMES (SEE SUBROUTINE FILEIO)              SIGMA1

----------------------------------------------------              SIGMA1

UNIT  FILE NAME                                                   SIGMA1

----  ----------                                                  SIGMA1

2   SIGMA1.INP                                                  SIGMA1

3   SIGMA1.LST                                                  SIGMA1

10   ENDFB.IN                                                    SIGMA1

11   ENDFB.OUT                                                   SIGMA1

12   (SCRATCH)                                                   SIGMA1

SIGMA1

INPUT CARDS                                                       SIGMA1

-----------                                                       SIGMA1

CARD  COLS.  DESCRIPTION                                          SIGMA1

----  -----  -----------                                          SIGMA1

1   1-11  SELECTION CRITERIA (0=MAT, 1=ZA)                     SIGMA1

12-22  MONITOR MODE SELECTOR                                SIGMA1

= 0 - NORMAL OPERATION                               SIGMA1

= 1 - MONITOR PROGRESS OF DOPPLER BROADENING OF DATA.SIGMA1

EACH TIME A PAGE OF DATA POINTS IS WRITTEN TO  SIGMA1

THE SCRATCH FILE PRINT OUT THE TOTAL NUMBER OF SIGMA1

POINTS ON SCRATCH AND THE LOWER AND UPPER      SIGMA1

ENERGY LIMITS OF THE PAGE (THIS OPTION MAY BE  SIGMA1

USED IN ORDER TO MONITOR THE EXECUTION SPEED   SIGMA1

OF LONG RUNNING JOBS).                         SIGMA1

23-33  KELVIN TEMPERATURE                                   SIGMA1

34-44  MINIMUM CROSS SECTION OF INTEREST                    SIGMA1

(DEFAULT VALUE = 1.0E-10 BARNS).                     SIGMA1

45-55  NEGATIVE CROSS SECTION TREATMENT                     SIGMA1

= 0 - O.K.                                           SIGMA1

= 1 - SET = 0                                        SIGMA1

56-66  UNRESOLVED RESONANCE REGION TREATMENT                SIGMA1

= 0 - COPY (NO BROADENING)                           SIGMA1

= 1 - IGNORE (BROADEN)                               SIGMA1

2   1-72  ENDF/B INPUT DATA FILENAME                           SIGMA1

(STANDARD OPTION = ENDFB.IN)                         SIGMA1

3   1-72  ENDF/B OUTPUT DATA FILENAME                          SIGMA1

(STANDARD OPTION = ENDFB.OUT)                        SIGMA1

4-N   1-11  LOWER MAT OR ZA LIMIT                                SIGMA1

12-22  UPPER MAT OR ZA LIMIT                                SIGMA1

UP TO 100 MAT OR ZA RANGES MAY BE SPECIFIED, ONE     SIGMA1

RANGE PER CARD. THE LIST OF RANGES IS TERMINATED BY  SIGMA1

A BLANK CARD. IF THE UPPER LIMIT IS LESS THAN THE    SIGMA1

LOWER LIMIT THE UPPER LIMIT WILL BE SET EQUAL TO THE SIGMA1

LOWER LIMIT. IF THE FIRST REQUEST CARD IS BLANK IT   SIGMA1

WILL TERMINATE THE LIST OF REQUESTS AND CAUSE ALL    SIGMA1

DATA TO BE RETRIEVED (SEE EXAMPLE INPUT).            SIGMA1

VARY  1-11  ENERGY FOR ERROR LAW                                 SIGMA1

12-22  ERROR FOR ERROR LAW                                  SIGMA1

THE ACCEPTABLE LINEARIZING ERROR CAN BE GIVEN AS AN  SIGMA1

ENERGY DEPENDENT FUNCTION SPECIFIED BY UP TO 20      SIGMA1

(ENERGY,ERROR) PAIRS AND LINEAR INTERPOLATION        SIGMA1

TABULATE POINTS. ENERGIES MUST BE IN ASCENDING ORDER.SIGMA1

THE ERROR LAW IS TERMINATED BY A BLANK CARD. IF THE  SIGMA1

FIRST ERROR LAW CARD IS BLANK IT WILL TERMINATE THE  SIGMA1

ERROR LAW AND THE ERROR WILL BE TREATED AS ENERGY    SIGMA1

INDEPENDENT, EQUAL TO ZERO, WHICH INDICATES THAT THE SIGMA1

BROADENED DATA SHOULD NOT BE THINNED.                SIGMA1

SIGMA1

EXAMPLE INPUT NO. 1                                               SIGMA1

-------------------                                               SIGMA1

BROADEN ALL URANIUM ISOTOPES AND THORIUM-232 TO 300 KELVIN. FROM  SIGMA1

0 TO 100 EV THIN OUTPUT DATA TO 0.1 PER-CENT ACCURACY. FROM 100 EVSIGMA1

TO 1 KEV VARY THE ERROR BETWEEN 0.1 AND 1 PER-CENT. ABOVE 1 KEV   SIGMA1

USE 1 PER-CENT ACCURACY.                                          SIGMA1

SIGMA1

EXPLICITLY SPECIFY THE STANDARD FILENAMES.                        SIGMA1

SIGMA1

THE FOLLOWING 11 CARDS ARE REQUIRED                               SIGMA1

SIGMA1

1          0 3.00000+ 2                                      SIGMA1

ENDFB.IN                                                              SIGMA1

ENDFB.OUT                                                             SIGMA1

92000      92999                                                 SIGMA1

90232               (UPPER LIMIT WILL AUTOMATICALLY BE DEFINED)  SIGMA1

(BLANK CARD INDICATES END OF REQUEST LIST)   SIGMA1

0.00000+ 0 1.00000-03                                                 SIGMA1

1.00000+ 2 1.00000-03                                                 SIGMA1

1.00000+ 3 1.00000-02                                                 SIGMA1

1.00000+ 9 1.00000-02                                                 SIGMA1

(BLANK CARD INDICATES END OF ERROR LAW)      SIGMA1

SIGMA1

EXAMPLE INPUT NO. 2                                               SIGMA1

-------------------                                               SIGMA1

BROADEN ALL DATA TO 300 KELVIN AND DO NOT THIN THE BROADEN DATA.  SIGMA1

ALL OF THE STANDARD OPTION MAY BE INVOKED MERELY BY SPECIFYING    SIGMA1

THE KELVIN TEMPERATURE ON THE FIRST CARD. ALL OTHER FIELDS MAY    SIGMA1

BE LEFT BLANK.                                                    SIGMA1

SIGMA1

LEAVE THE DEFINITION OF THE FILENAMES BLANK - THE PROGRAM WILL    SIGMA1

THEN USE STANDARD FILENAMES.                                      SIGMA1

SIGMA1

THE FOLLOWING 5 CARDS ARE REQUIRED                                SIGMA1

SIGMA1

3.00000+ 2                                      SIGMA1

(USE STANDARD FILENAME = ENDFB.IN)              SIGMA1

(USE STANDARD FILENAME = ENDFB.OUT)             SIGMA1

(RETRIEVE ALL DATA, TERMINATE REQUEST LIST)     SIGMA1

(0.0 ALLOWABLE ERROR, TERMINATE ERROR LAW)      SIGMA1

SIGMA1

EXAMPLE INPUT NO. 3                                               SIGMA1

-------------------                                               SIGMA1

THE SAME AS ABOVE, ONLY DEFINE THE MINIMUM CROSS SECTION OF       SIGMA1

INTEREST TO BE 1.0E-30 BARNS (INSTEAD OF THE DEFAULT VALUE OF     SIGMA1

1.0E-10).                                                         SIGMA1

SIGMA1

READ ENDF/B DATA FROM \ENDFB6\RECENT\ZA092238 AND WRITE ENDF/B    SIGMA1

DATA TO \ENDFB\SIGMA1\ZA092238                                    SIGMA1

SIGMA1

THE FOLLOWING 5 CARDS ARE REQUIRED                                SIGMA1

SIGMA1

3.00000+ 2 1.00000-30                           SIGMA1

\ENDFB6\RECENT\ZA092238                                               SIGMA1

\ENDFB6\SIGMA1\ZA092238                                               SIGMA1

(RETRIEVE ALL DATA, TERMINATE REQUEST LIST)     SIGMA1

(0.0 ALLOWABLE ERROR, TERMINATE ERROR LAW)      SIGMA1

SIGMA1

=======================================================================SIGMA1